Neural Networks on FPGAs

Final Project — Student Instructions

Submission Deadline: 10 April 2025

# 1. Project Overview

Congratulations on reaching the final project! This is your chance to put everything you have learned together into something real — a neural network that actually runs on hardware.

Do not be intimidated. This document walks you through every step in simple, clear language. You will implement a small neural network in Verilog, generate its weights using Python, synthesise the design in Vivado, and finally run it on your FPGA board. Take it one day at a time.

What you will build A 2-layer neural network implemented in Verilog that classifies simple inputs. Weights and biases trained in Python (TensorFlow) and loaded onto the FPGA. A working simulation that proves your design is correct before touching any hardware. A short report describing what you did and what you observed.

# 2. How to Submit

Submission Deadline & Location Submit everything on GitHub by 11:59 PM on 10 April 2025. Create your folder at this exact path inside the repository: ends_term/project/yourname_rollno/ Example: ends_term/project/mananjindal_230625/ Use lowercase. Replace 'yourname' with your first name and 'rollno' with your roll number. Do NOT submit a zip file. Commit and push your files directly to GitHub.

## 2.1 Your Folder Structure

Create the following folders inside your submission folder. You do not need to fill them all on Day 1 — just create them and add files as you go.

ends_term/project/mananjindal_230625/

src/          <- Your Verilog files go here

sim/          <- Your testbench files go here

weights/      <- The .mem files generated by Python

python/       <- Your Python training script

vivado/       <- Your Vivado project files

docs/         <- Your report (PDF)

README.md     <- A short description of your project

Tip: Make your first GitHub commit today with just the empty folders and a README. Committing regularly (every day) is better than one big commit on the last night.

# 3. Week Plan

Here is a suggested day-by-day breakdown. You submit everything together on 10 April, but sticking to this schedule will keep you from panicking at the end.

| Phase | Task | Day |
| --- | --- | --- |
| Day 1 | Set up Vivado, create your GitHub folder, push your first commit with a README. | 1 |
| Day 2 | Run the Python script to train a tiny model and export weights/biases as .mem files. | 2 |
| Day 3 | Write the neuron module in Verilog and simulate it. Make sure it works before moving on. | 3 |
| Day 4 | Build a single layer by connecting neurons together. Test with a simulation. | 4 |
| Day 5 | Combine both layers into a top-level module. Run Synthesis in Vivado. | 5 |
| Day 6 | Flash the bitstream to your board and verify a few test inputs give the right output. | 6 |
| Day 7 | Write your report, tidy up your code and README, and push everything to GitHub. | 7 |

# 4. Step-by-Step Instructions

## Task 1: Setup (Day 1)

Before any coding, get your tools and repository ready.

- Open Vivado. Create a new RTL project (you can use a dummy top module for now just to verify it opens). If Vivado is not installed, follow the installation guide shared by your mentor.

- On GitHub, navigate to the project repository. Inside it, create your personal folder: ends_term/project/yourname_rollno/. Add the sub-folders listed in Section 2.1.

- Write a README.md. It only needs three lines right now: your name, your roll number, and one sentence describing what this project is. You will fill in the rest at the end.

- Commit and push. That is it for Day 1.

## Task 2: Python — Train the Model and Export Weights (Day 2)

You do not need to be a machine learning expert for this step. You will train a very small model on a simple dataset and save the numbers it learns (weights and biases) into files that Verilog can read.

### What model to train

Use the Iris dataset (built into scikit-learn and TensorFlow datasets). Train a 2-layer network with this exact structure:

- Input: 4 features

- Hidden layer: 8 neurons, ReLU activation

- Output layer: 3 neurons, Softmax (3 flower classes)

### Quantising the weights (converting floats to integers)

FPGAs work with integers, not decimals. After training, convert each weight and bias from a float to a 16-bit integer using this simple formula: multiply by 256 and round. This is called Q8 fixed-point format and is easier to work with than the Q4.12 format used in advanced designs.

# Example: convert a single weight

quantised = int(round(float_weight * 256))

quantised = max(-32768, min(32767, quantised))  # keep it in 16-bit range

### Saving the .mem files

Save weights, biases, and 10 test inputs into text files in hex format, one number per line. Name them:

- weights/weights.mem

- weights/biases.mem

- weights/test_data.mem  (10 input vectors + expected output label each)

Save your script as python/train_and_export.py. Running it should regenerate all three files from scratch.

Stuck on Python? Your mentor can provide a starter script with the model already defined. You just need to fill in the export section that saves the .mem files.

## Task 3: Verilog — The Neuron Module (Day 3)

A neuron does one thing: multiply each input by its weight, add them all up, add a bias, then pass the result through an activation function (ReLU). This is called a Multiply-Accumulate operation, or MAC.

You only need to write ONE Verilog module for this task: the neuron itself. We keep it simple — no pipeline, no FIFO, just a straightforward sequential design.

### Module interface

Create src/neuron.v with the following ports:

module neuron (

input  wire        clk,

input  wire        rst_n,      // active-low reset

input  wire        start,      // pulse high for 1 cycle to begin

input  wire [15:0] data_in,    // one input value at a time (Q8 format)

input  wire [15:0] weight_in,  // matching weight for that input

input  wire [15:0] bias,       // bias value

input  wire        last,       // pulse high on the final input

output reg  [15:0] out,        // result after ReLU

output reg         valid       // high for 1 cycle when output is ready

);

### What the module should do

- On each clock cycle, multiply data_in by weight_in and add the result to an internal accumulator register.

- When last goes high, add the bias to the accumulator and apply ReLU: if the result is negative, output 0; otherwise output the result (truncated to 16 bits).

- Assert valid high for one cycle when the output is ready.

### ReLU in Verilog

// Apply ReLU: output 0 if negative, else output the value

assign relu_out = (sum[31]) ? 16'd0 : sum[23:8]; // use bits [23:8] for Q8 truncation

### Testbench

Write sim/tb_neuron.v. Drive the neuron with 4 known inputs and weights (you can calculate the expected output by hand or with Python). Check that valid goes high and that out matches the expected value. Use $display to print results and $finish to end the simulation.

Do NOT move to Task 4 until your simulation passes. A working neuron testbench is the most important checkpoint in this project.

## Task 4: Verilog — One Layer (Day 4)

A layer is just multiple neurons. For this project, you will implement one hidden layer with 8 neurons, all processing the same input in parallel (one neuron per output).

### How to build it

Create src/layer.v. Inside it, instantiate 8 copies of your neuron module (one for each output). Each neuron gets the same data_in and start signals, but a different set of weights from the .mem file.

Load the weights using $readmemh. Here is how to declare a weight memory and load it:

reg [15:0] weight_mem [0:31]; // 8 neurons x 4 inputs = 32 weights

initial $readmemh("weights/weights.mem", weight_mem);

Then connect each neuron's weight_in to the right entry in weight_mem based on the current input index.

### Layer testbench

Write sim/tb_layer.v. Feed one complete 4-value input vector into the layer and check that all 8 output values become valid and look reasonable (positive numbers for active neurons, 0 for inactive ones after ReLU).

## Task 5: Top-Level Integration and Synthesis (Day 5)

### 5a. Top-level module

Create src/nn_top.v that connects your hidden layer (8 outputs) to a simpler output layer (3 neurons — one per class). The output neuron with the highest value is the predicted class.

Add two control signals: a start input that kicks off inference, and a done output that goes high when the result is ready.

### 5b. Running Synthesis in Vivado

- Create a new Vivado project. Select your target board (e.g. Basys3 or Nexys A7).

- Add all .v files from src/ to the project.

- Add a constraints file (.xdc). You only need one line in it: the clock constraint.

create_clock -period 10.000 [get_ports clk]  ;# 100 MHz clock

- Click Run Synthesis. Wait for it to complete (this takes a few minutes).

- If you see red errors, fix them. Yellow warnings about unused signals are usually fine.

- Click Run Implementation. When done, click Generate Bitstream.

- Save the Vivado project files into your vivado/ folder and commit.

Common beginner synthesis errors: - 'Latch inferred' -> make sure every output is assigned in all branches of an if/else - 'Width mismatch' -> check that you are not connecting a 16-bit wire to an 8-bit port These are easy to fix once you read the error message carefully.

## Task 6: Hardware and Basic Verification (Day 6)

### 6a. Programming the board

- Open Vivado Hardware Manager (Flow Navigator → Open Hardware Manager).

- Connect your FPGA board via the USB cable and click Auto Connect.

- Click Program Device and select your .bit file.

- The board should power up. If you have an LED connected to your done signal, it should blink when inference finishes.

### 6b. Verify your results

Pick 5 test inputs from your test_data.mem file. Using switches or a UART interface (whichever your board setup uses), feed each input to the FPGA and note the output. Compare it to the expected label. Record your observations in a simple table like this:

| Test # | Input Values | Expected Class | FPGA Output |
| --- | --- | --- | --- |
| 1 | [ _, _, _, _ ] | Class _ | Class _ |
| 2 | [ _, _, _, _ ] | Class _ | Class _ |
| 3 | [ _, _, _, _ ] | Class _ | Class _ |
| 4 | [ _, _, _, _ ] | Class _ | Class _ |
| 5 | [ _, _, _, _ ] | Class _ | Class _ |

### 6c. Two metrics to record (that is all!)

You only need to report two numbers. Find them in Vivado after synthesis completes:

- LUT utilisation — open the Utilisation Report in Vivado and find the percentage of LUTs used. Write it down.

- Timing slack (WNS) — open the Timing Summary Report. Find 'Worst Negative Slack'. If it is a positive number, your design meets timing. Write it down.

That is all the metrics you need. Include these two numbers in your report and README.

## Task 7: Report and Final Cleanup (Day 7)

### 7a. Written Report

Write a short report (2–3 pages is fine) saved as docs/report.pdf. Do not overthink it — just answer these questions in your own words:

- What does your design do? (1 short paragraph)

- Draw a simple block diagram showing the neuron, layer, and top-level module connected together.

- What results did you get? (paste your 5-test verification table and the 2 metrics from Task 6)

- What was the hardest part for you? What did you learn?

### 7b. Complete your README.md

Update your README.md to include:

- Your name and roll number

- Your FPGA board name

- A one-paragraph description of the project

- How to run the Python script (one command)

- How to open and program in Vivado (2–3 sentences)

- Your 2 metrics: LUT utilisation and timing slack

# 5. Final Checklist Before Submitting

Go through this list before your last push. If something is missing, add it.

| File | Description | Notes |
| --- | --- | --- |
| README.md | Name, roll no, board, description, metrics | Required |
| src/neuron.v | Single neuron module (MAC + ReLU) | Required |
| src/layer.v | One hidden layer (8 neurons instantiated) | Required |
| src/nn_top.v | Top-level connecting hidden + output layer | Required |
| sim/tb_neuron.v | Neuron testbench — must show correct output | Required |
| sim/tb_layer.v | Layer testbench | Required |
| python/train_and_export.py | Python script that generates .mem files | Required |
| weights/*.mem | weights.mem, biases.mem, test_data.mem | Required |
| vivado/ | Vivado project files (.xpr) | Required |
| docs/report.pdf | 2–3 page report with block diagram and results | Required |

# 6. Grading Breakdown

| Component | Marks | Notes |
| --- | --- | --- |
| Python script + correct .mem files generated | 15 | Script must run without errors |
| Neuron module + passing testbench simulation | 25 | Simulation must show correct output |
| Layer module (8 neurons, weights loaded from .mem) | 20 | Must synthesise cleanly in Vivado |
| Top-level integration + bitstream generated | 15 | Bitstream must be present in vivado/ |
| Hardware verification (5 test inputs checked) | 10 | Fill in the table from Task 6 |
| Report (block diagram + results + reflection) | 10 | 2–3 pages, honest and clear |
| README + code comments + commit history | 5 | Shows understanding of the work |
| Total | 100 |  |

# 7. Rules

- Write your own Verilog. Do not copy-paste full modules from the internet or from classmates.

- You can discuss ideas and debug together, but what you submit must be written by you.

- You can use AI tools (ChatGPT, etc.) to understand a concept or debug an error message, but you must understand and be able to explain every line you submit.

- Late submissions lose 10 marks per day.

# 8. Getting Help

Getting stuck is normal and expected. Here is how to get help quickly:

- Simulation not working? Post your tb_neuron.v code and the exact error in the group chat. Do not post screenshots of code — paste the text.

- Vivado synthesis error? Copy the full error message text and search it. Most errors have a clear fix.

- Hardware not responding? Double-check that you have correctly set the clock constraint and that the bitstream was generated after your last code change.

- Not sure about the .mem file format? Ask your mentor for a sample — seeing one correct example is worth more than reading the documentation.

How to reach your mentor Open a GitHub Issue in the project repository for questions that might help the whole class. For personal help, message your mentor directly and include a link to your GitHub folder. Always describe: (1) what you tried, (2) what you expected, (3) what actually happened.

You have got this. One task at a time.